# Project Proposal

Title: VR-Guided Target Pose Control with Autonomous Whole-body Stabilization for Humanoid Robots

# 1. Background

Humanoid robot teleoperation traditionally relies either on full-body motion retargeting or low-level joint control. This increases operator workload and often produces unstable motions because humans are required to explicitly control locomotion and balance. This project proposes a hierarchical teleoperation framework where the operator specifies task intentions through immersive VR, while the robot autonomously generates dynamically feasible whole-body motions.

# 2. Research Objective

The objective is to develop a VR-based teleoperation framework that combines direct control, command control, visual perception, and autonomous whole-body stabilization. The operator controls only the head and both hands continuously, while discrete commands are provided through speech and gesture. The humanoid robot autonomously plans locomotion, balance, and whole-body coordination to achieve the desired target pose.

# 3. System Architecture

The framework consists of six modules:
1) Human Interaction Layer (VR headset, controllers, speech, gesture).
2) Target Pose Manager (fusion of continuous poses and discrete commands).
3) Vision Perception Layer (object detection, scene understanding, traversability, reachability, affordance estimation).
4) Autonomous Whole-body Motion Planner (whole-body IK, footstep planning, collision avoidance, balance planning).
5) Whole-body Controller (WBC/MPC for dynamically feasible execution).
6) Immersive Feedback Layer (robot first-person view with augmented robot state information).

# 4. Control Strategy

Direct Control:
- Continuous control of head pose.
- Continuous control of left and right hand poses.

Command Control:
- Speech commands (walk, turn, squat, stand, grasp, navigate).
- Gesture commands (select target, confirm, release, mode switching).

Autonomous Control:
- Whole-body stabilization.
- Footstep generation.
- Pelvis and torso adjustment.
- Collision avoidance.
- Contact planning.
- Dynamic balance control.

The operator specifies WHAT should be achieved, while the robot determines HOW to achieve it.

# 5. Project Workflow

Stage 1: VR hardware integration and robot communication.
Stage 2: Target pose generation from VR tracking.
Stage 3: Speech and gesture recognition.
Stage 4: Vision perception and environment modeling.
Stage 5: Autonomous whole-body planning.
Stage 6: Whole-body control on the humanoid robot.
Stage 7: Closed-loop VR feedback and evaluation.

# 6. Expected Contributions

• A hierarchical VR teleoperation framework for humanoid robots.
• Target-pose-driven shared autonomy.
• Vision-guided autonomous whole-body stabilization.
• Reduced operator workload while maintaining precise manipulation.
• A unified immersive interface integrating teleoperation, perception, and autonomous motion generation.

# 7. Evaluation

Experiments will compare:
- Pure VR teleoperation.
- Full-body motion retargeting.
- Proposed hierarchical framework.

Metrics include task completion rate, operation time, end-effector accuracy, robot stability, number of falls, operator workload (NASA-TLX), and user experience.

# 8. Conclusion

The proposed framework introduces a new teleoperation paradigm in which immersive VR provides target poses and high-level task intentions, while the humanoid robot autonomously generates stable whole-body motions using vision-guided perception and whole-body control. This enables intuitive operation, improved safety, and scalable deployment for complex humanoid manipulation tasks.